MM-1200 #

For High RF Field:

Any machine used in a high RF field (all of NYC) should have a 1 uH choke

placed in series with the base of Q1 on each Reproduce PWA.  Add .0033

microfarad cap (polystyrene) to base of Q2 to ground.

This is especially effective against TV interference common in New York.

Line Amp Gain Reduction:

Change R18 to 20K ohm (was 560K) on repro card.

This will reduce pops & clicks in monitors, but will require realignment

of rec/rep/sync.

#